SURAKSHA CROWDVISION

K3s Edge Orchestration, Dual RTX A4000 GPU Sharding & Deployment Runbook

| Document Title: | Suraksha CrowdVision: K3s Deployment Architecture & Operations Runbook |
| --- | --- |
| Version & Status: | Version 2.0 (Production Verified — Dual GPU Sharding & 28 Cameras) |
| Hardware Workstation: | HP Z6 G4 Workstation (Intel Xeon 48 vCPUs, 128 GB RAM, 2x NVIDIA RTX A4000) |
| Orchestration Platform: | K3s Lightweight Kubernetes (v1.30+), containerd, Traefik Ingress Controller |
| Operating System & Driver: | Ubuntu 24.04 LTS (Kernel 6.8), NVIDIA Driver 580.178.04, CUDA 13.0 / 12.x |
| Author / Organization: | Tride Mobility AI Engineering Team (saicharananandapu@tridemobility.com) |

1. Executive Overview & System Architecture

Suraksha CrowdVision is an edge-deployed, multi-camera computer vision and passenger density monitoring platform operating at major Indian Railway hubs. The application continuously processes 28 live high-definition RTSP video feeds across foot-over-bridges (FOBs), platform concourses, stairwells, and waiting halls to compute real-time passenger counts, spatial heatmaps, congestion alerts, and stampede risk scores.

To satisfy strict edge operational constraints—including zero external cloud dependency for inference, complete power-loss recovery, dual hardware GPU acceleration, and low-latency local streaming—the platform is containerized and orchestrated on a single high-performance edge workstation running K3s (Lightweight Kubernetes).

Key Architecture Principles: • Workload Sharding: 28 cameras are divided evenly (14 cameras each) across two independent NVIDIA RTX A4000 GPUs. • Deterministic Stateful Orchestration: Backend uses a Kubernetes StatefulSet with dedicated GPU device binding. • Unified Ingress Routing: Traefik Ingress routes web, API, Swagger, and WebSocket connections over port 80/443. • Continuous Availability: Fully configured auto-start chain ensures unattended recovery within 60s of power restoration.

Figure: Suraksha CrowdVision End-to-End K3s Cluster Architecture & Network Topology

2. Edge Workstation Hardware & Compute Profile

The edge workstation running CrowdVision is an enterprise-grade HP Z6 G4 Workstation engineered for continuous 24/7 compute loads. The hardware is configured with dual high-bandwidth NVIDIA RTX A4000 GPUs, massive system memory, and enterprise NVMe storage to guarantee zero thermal throttling and stable inference frame rates.

| Hardware Subsystem | Physical Specification | Operational Role in CrowdVision |
| --- | --- | --- |
| Host Processor (CPU) | Intel Xeon Platinum 8260 (24 Cores / 48 vCPUs @ 2.40 GHz) | Decoupled OpenCV RTSP decoding, video frame buffers, FastAPI serving |
| System Memory (RAM) | 128 GB DDR4 2933 MHz ECC Registered (125 GiB Usable) | Host frame ring buffers, in-memory rolling metrics (Zero-disk daemon) |
| GPU Accelerator #0 | NVIDIA RTX A4000 (16 GB GDDR6 ECC, 6144 CUDA, PCIe 21:00.0) | Assigned to Shard 0 (crowdvision-backend-0): 14 Camera Streams |
| GPU Accelerator #1 | NVIDIA RTX A4000 (16 GB GDDR6 ECC, 6144 CUDA, PCIe 2D:00.0) | Assigned to Shard 1 (crowdvision-backend-1): 14 Camera Streams |
| Storage Subsystem | 2 TB NVMe PCIe Gen4 M.2 SSD + Secondary Data Drive | Stores OS, K3s containerd local images, YOLO weights, homography zones |
| Network Interfaces | Dual Intel Gigabit NICs (Active Node IP: 10.54.23.55) | RTSP camera ingestion from station LAN + Traefik dashboard serving |
| Power & Cooling | 1000W 90% Efficient Chassis Power Supply + Active Blowers | Independent dual GPU blower exhaust maintaining operating temps <95°C |

3. Dual GPU & CPU Resource Utilization Configurations

A critical engineering requirement of Suraksha CrowdVision is deterministic hardware isolation. If all 28 video streams competed for a single GPU, CUDA memory allocation would saturate immediately, leading to Out-Of-Memory (OOM) kernel panics and dropped frames. The solution employs K3s StatefulSet pod sharding backed by the NVIDIA Container Runtime.

3.1: Workload Partitioning & StatefulSet Sharding Mechanism

Instead of a standard Kubernetes Deployment (which launches interchangeable, non-indexed replicas), the backend uses a StatefulSet named `crowdvision-backend` with `replicas: 2` and `podManagementPolicy: Parallel`.

Shard 0: Pod crowdvision-backend-0: Pinned to Hardware GPU 0 via NVIDIA Container Toolkit.

Shard 1: Pod crowdvision-backend-1: Pinned to Hardware GPU 1 via NVIDIA Container Toolkit.

Policy Parallel: Parallel Startup: Both pods boot concurrently without waiting for sequential ordering.

Even Distribution: Camera Modulo Partitioning: Each pod determines its active camera list using (Camera_Index % 2 == Shard_ID).

3.2: Pod Resource Allocation Matrix (Requests & Limits)

Resource boundaries are strictly configured in the pod specification to guarantee that background processes do not starve the operating system while preventing memory leaks from crashing the host node.

| Component Pod | CPU Request | CPU Limit | Memory Request | Memory Limit | GPU Binding |
| --- | --- | --- | --- | --- | --- |
| crowdvision-backend-0 | 8 vCPUs | 17 vCPUs | 8 GiB | 24 GiB | 1x GPU (NVIDIA RTX A4000 #0) |
| crowdvision-backend-1 | 8 vCPUs | 17 vCPUs | 8 GiB | 24 GiB | 1x GPU (NVIDIA RTX A4000 #1) |
| crowdvision-frontend | 0.5 vCPUs | 2.0 vCPUs | 256 MiB | 1.0 GiB | None (CPU Nginx Static Engine) |
| mediamtx | 1.0 vCPUs | 4.0 vCPUs | 512 MiB | 2.0 GiB | None (RTSP Network Streaming) |

3.3: GPU VRAM Memory Budget Breakdown

Each NVIDIA RTX A4000 provides 16,376 MiB of high-speed GDDR6 VRAM. The application memory allocation per GPU is dimensioned as follows:

CUDA Context: PyTorch Base CUDA Context: ~450 MiB overhead upon initialization.

YOLOv8 Weights: YOLOv8 Person & Head Model: ~1,850 MiB for FP16 inference engine and TensorRT buffers.

Tracking Engine: ByteTrack / Re-ID Embedding Model: ~1,200 MiB for feature extraction across multiple cameras.

Frame Queues: Active Ingestion Frame Buffers: ~3,200 MiB for 14 concurrent circular frame buffers.

Tensor Workspace: Batched Inference Tensors: ~7,500 MiB dynamic scratchpad memory during peak batching.

Measured Utilization: Active Memory Usage: ~13.6 GiB (GPU 0) and ~15.7 GiB (GPU 1), operating reliably within hardware capacity.

Figure: Dual NVIDIA RTX A4000 GPU & CPU Resource Utilization Architecture

4. K3s Networking & Ingress Routing Topology

K3s bundles Traefik as its standard Ingress controller, listening directly on the host network interfaces (port 80). The ingress configuration routes HTTP requests to the appropriate in-cluster service based on URL path prefixes.

| Path Prefix | In-Cluster Target Service | Target Port | Description & Client Audience |
| --- | --- | --- | --- |
| / | crowdvision-frontend | 80 (HTTP) | Serves React SPA dashboard, static JS/CSS assets, FOB SVG map |
| /api | crowdvision-backend | 8000 (HTTP) | REST API routes for cameras, zones, counts, alerts, analytics |
| /docs | crowdvision-backend | 8000 (HTTP) | Interactive OpenAPI Swagger documentation for API consumers |
| /openapi.json | crowdvision-backend | 8000 (HTTP) | Raw OpenAPI JSON schema definition |

Cross-Subnet Access via DNS Name vs IP Address: • Local IP Address (http://10.54.23.55/): Fully routable across all corporate subnets and VLANs. • mDNS Domain (http://crowdvision-dashboard.local): Resolves automatically within the local subnet (10.54.23.0/24). • Cross-Subnet Recommendation: For devices on other office subnets, configure a corporate DNS A-record pointing 'crowdvision.tride.live' or 'crowdvision-dashboard' to 10.54.23.55.

5. End-to-End Step-by-Step Deployment Runbook

This section provides the complete, copy-paste terminal runbook for building, importing, deploying, and rolling out CrowdVision on K3s from scratch.

Figure: End-to-End Application Deployment & Zero-Downtime Rollout Pipeline

Phase 1: Environment Preparation & Git Code Checkout

Verify working directory and ensure target branches are synchronized:

# Navigate to the deployment repository cd /home/user/k3s/Suraksha-deployment # Verify git status and active branch git status git branch # Sync backend and frontend code to working directory # Backend source: https://github.com/TrideAdmin/crowd-backend.git (branch: videotest) # Frontend source: https://github.com/TrideAdmin/Crowd_Vision_Frontend.git (branch: master)

Phase 2: Building Local Container Images

Build Docker container images locally for both the frontend and backend:

# 1. Build Frontend Container Image docker build \ --build-arg "VITE_API_URL=/api" \ -t crowdvision-frontend:local \ /home/user/k3s/Suraksha-deployment/Crowd_Vision_Frontend # 2. Build Backend Container Image (includes CUDA & PyTorch) docker build \ -t crowdvision-backend:local \ /home/user/k3s/Suraksha-deployment/crowd-backend # Verify built images docker images ¦ grep crowdvision

Phase 3: Ingesting Images into K3s containerd Runtime

Export Docker images and import directly into the K3s containerd `k8s.io` namespace:

# Export and import Frontend image into K3s containerd docker save crowdvision-frontend:local ¦ sudo k3s ctr -n k8s.io images import - # Export and import Backend image into K3s containerd docker save crowdvision-backend:local ¦ sudo k3s ctr -n k8s.io images import - # Verify images are recognized by K3s containerd sudo k3s ctr -n k8s.io images list ¦ grep crowdvision

Phase 4: Creating Kubernetes Namespace, Secrets & ConfigMaps

Apply namespace and generate application secrets from the production `.env` file:

# 1. Create crowdvision namespace kubectl create namespace crowdvision --dry-run=client -o yaml ¦ kubectl apply -f - # 2. Apply Application ConfigMap kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/configmap.yaml # 3. Create Secrets from backend .env file (MongoDB, SMTP, API keys) kubectl -n crowdvision create secret generic crowdvision-secrets \ --from-env-file=/home/user/k3s/Suraksha-deployment/crowd-backend/.env \ --dry-run=client -o yaml ¦ kubectl apply -f - # Verify Secrets created kubectl get secrets -n crowdvision

Phase 5: Deploying Workloads (Storage, MediaMTX, Frontend & Backend)

Apply PersistentVolumeClaims, video proxy, frontend, backend StatefulSet, and Ingress:

# 1. Apply Persistent Storage Claim for AI Weights kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/pvc.yaml # 2. Deploy MediaMTX Video Streaming Proxy kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/mediamtx-deployment.yaml kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/mediamtx-service.yaml # 3. Deploy Frontend Web Application kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/stack/base/frontend.yaml # 4. Deploy Headless Service for StatefulSet kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/backend-headless-service.yaml kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/backend-service.yaml # 5. Deploy Backend StatefulSet (GPU Sharding) kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/backend-statefulset.yaml # 6. Apply Traefik Ingress Rules kubectl apply -f /home/user/k3s/Suraksha-deployment/k3s/ingress.yaml

Phase 6: Triggering Rolling Restarts & Verifying Status

Perform zero-downtime rolling restart to guarantee all pods load the newest images:

# Rolling restart of Frontend deployment kubectl rollout restart deployment crowdvision-frontend -n crowdvision # Rolling restart of Backend StatefulSet kubectl rollout restart statefulset crowdvision-backend -n crowdvision # Watch rollout status kubectl rollout status statefulset/crowdvision-backend -n crowdvision kubectl rollout status deployment/crowdvision-frontend -n crowdvision # Check running pods and resource allocations kubectl get pods -n crowdvision -o wide

6. AI Video Analytics & Camera Fleet Partitioning

The station monitoring deployment encompasses 28 active cameras configured in the database `crowdvision_prod`. The inference pipeline decouples stream grabbing from deep learning inference to prevent network stalls from blocking the GPU.

Figure: AI Video Analytics & Computer Vision Inference Pipeline

6.1: Active Camera Fleet Partitioning (Shard 0 vs Shard 1)

Cameras are assigned to specific pods and GPUs as shown in the table below:

| Stream # | Shard 0 (GPU 0 — crowdvision-backend-0) | Shard 1 (GPU 1 — crowdvision-backend-1) |
| --- | --- | --- |
| 1 | cam_middle_fob_4_5 (Middle FOB Platform 4-5) | cam_kzj_fob_mid_4_5 (KZJ FOB Middle 4-5) |
| 2 | cam_pf1_fob_hyb_end (PF 1 FOB HYB End) | cam_kzj_fob_mid_8_9 (KZJ FOB Middle 8-9) |
| 3 | cam_kzj_pf1_fob_kzj (KZJ PF 1 FOB) | cam_pf8_mid_fc_kzj (PF 8 Middle FC KZJ) |
| 4 | cam_kzj_pf1_fob_pf10 (KZJ PF 1 to PF 10) | cam_gate2_wh (Gate 2 Waiting Hall) |
| 5 | cam_mid_fob_pf1 (Middle FOB PF 1) | cam_hyd_booking_gate2a (HYD Booking Gate 2A) |
| 6 | cam_mid_fob_center (Middle FOB Center Concourse) | cam_near_gate_2a_fc_swathi_ent (Near Gate 2A Swathi) |
| 7 | cam_hyb_pf1 (HYB Platform 1) | cam_gate2_fc_ac_wh (Gate 2 FC AC Waiting Hall) |
| 8 | cam_hyb_pf2 (HYB Platform 2) | cam_gate2a_fc_parking (Gate 2A FC Parking Area) |
| 9 | cam_hyb_pf4 (HYB Platform 4) | cam_pf1_fob_pf10 (PF 1 FOB to PF 10) |
| 10 | cam_hyb_pf10 (HYB Platform 10) | cam_new_kzj_fob_fc_pf1 (New KZJ FOB FC PF 1) |
| 11 | cam_hyb_pf1_a (HYB Platform 1A) | cam_new_kzj_fob_near_pf1 (New KZJ FOB Near PF 1) |
| 12 | cam_hyb_booking (HYB Main Booking Hall) | cam_pf10_bme_counter (PF-10 Waiting Hall Area) |
| 13 | cam_hyb_booking_gate4a (HYB Booking Gate 4A) | cam_hyb_booking_gate6 (HYB Booking Gate 6) |
| 14 | cam_pf2_fc_hyd_side (PF 2 FC HYD Side) | cam_hyb_booking_gate8 (HYB Booking Gate 8) |

PF-10 Waiting Hall Area Integration: Camera 'cam_pf10_bme_counter' maps directly to zone 'zone_pf10_waiting_hall' in MongoDB crowdvision_prod. The frontend Dashboard renders the live crowd count badge (streaming at 45-52 pax) directly in the camera stream card header with real-time WebSocket updates.

7. Operations, Monitoring & Health Verification Commands

Use these operational commands to inspect cluster health, GPU activity, and pod logs in real time:

7.1: Kubernetes Cluster & Pod Telemetry

# Check all resources across the crowdvision namespace kubectl get all -n crowdvision # Inspect pod resource consumption (CPU and Memory) kubectl top pods -n crowdvision # Check detailed pod events and describe status kubectl describe pod crowdvision-backend-0 -n crowdvision # Stream live backend inference logs (tail last 50 lines) kubectl logs -n crowdvision -l app.kubernetes.io/name=crowdvision-backend -f --tail=50 # Stream frontend access logs kubectl logs -n crowdvision -l app.kubernetes.io/name=crowdvision-frontend -f --tail=50

7.2: NVIDIA Hardware GPU Live Monitoring

# Check current GPU temperature, VRAM usage, and compute utilization nvidia-smi # Watch GPU telemetry refresh every 1 second watch -n 1 nvidia-smi # Query detailed GPU temperature and power metrics nvidia-smi --query-gpu=index,name,temperature.gpu,utilization.gpu,memory.used,memory.total --format=csv

7.3: In-Memory Overnight Monitor Daemon (Zero Local Files)

# Check status of the 3-hour reporting daemon systemctl status crowdvision-monitor.service # Stream live monitor daemon journal logs (without disk clutter) journalctl -u crowdvision-monitor.service -f # Restart the monitor daemon sudo systemctl restart crowdvision-monitor.service

8. Auto-Start & Power-Loss Recovery Configuration

To ensure continuous availability in station environments subject to electrical maintenance or power outages, the workstation is configured with a fully automated, unattended startup chain.

Figure: Hardware Auto-Boot & High-Availability Service Recovery Chain

8.1: Hardware BIOS Configuration (Power-Cut Auto-Boot)

To enable the HP Z6 G4 Workstation to power on automatically when electricity returns:

Step 1: Power on the machine and repeatedly press F10 during boot to enter BIOS Setup.

Step 2: Navigate to: Advanced -> Power Management Options -> After Power Loss.

Step 3: Change the value from 'Power Off' to 'Power On' (or 'Previous State').

Step 4: Press F10 to Save Changes and Exit. The system will now boot automatically upon power connection.

8.2: Software systemd Auto-Start Chain

Verify that all system services are enabled to launch automatically at boot:

# Verify and enable MongoDB service sudo systemctl enable mongod.service # Verify and enable K3s orchestration engine sudo systemctl enable k3s.service # Verify and enable mDNS domain publisher service systemctl --user enable crowdvision-mdns.service sudo loginctl enable-linger user # Verify and enable Overnight Monitor service sudo systemctl enable crowdvision-monitor.service

9. Troubleshooting & Operational Resolutions

Issue: GPU Out-Of-Memory (OOM) / CUDA Allocation Error Cause: An un-sharded deployment was applied where both replicas attempted to bind to GPU 0. Fix: Verify that backend-statefulset.yaml contains 'CAMERA_SHARD_COUNT=2' and that each pod replica properly identifies its pod index from metadata.name.

Issue: ErrImageNeverPull / ImagePullBackOff Cause: The pod has 'imagePullPolicy: Never' but the container image was not imported into K3s containerd. Fix: Run 'docker save crowdvision-backend:local ¦ sudo k3s ctr -n k8s.io images import -' and verify with 'sudo k3s ctr -n k8s.io images list ¦ grep crowdvision'.

Issue: Duplicate Environment Variables in Secrets Cause: Multiple definitions of SMTP_PASSWORD or MONGODB_URI in the .env file cause kubectl create secret to fail. Fix: Comment out redundant legacy credentials in .env, keeping only the active production configuration.

10. Appendix: Complete Production K3s Manifests

10.1: StatefulSet Manifest (k3s/backend-statefulset.yaml)

apiVersion: apps/v1 kind: StatefulSet metadata: name: crowdvision-backend labels: app.kubernetes.io/name: crowdvision-backend spec: serviceName: crowdvision-backend-headless replicas: 2 podManagementPolicy: Parallel selector: matchLabels: app.kubernetes.io/name: crowdvision-backend template: metadata: labels: app.kubernetes.io/name: crowdvision-backend spec: runtimeClassName: nvidia terminationGracePeriodSeconds: 60 containers: - name: backend image: crowdvision-backend:local imagePullPolicy: Never ports: - name: http containerPort: 8000 env: - name: CAMERA_SHARD_COUNT value: "2" - name: HEADLESS_SERVICE_NAME value: "crowdvision-backend-headless" - name: HEADLESS_SERVICE_PORT value: "8000" - name: NVIDIA_DRIVER_CAPABILITIES value: "compute,utility,video" - name: POD_NAME valueFrom: fieldRef: fieldPath: metadata.name envFrom: - configMapRef: name: crowdvision-config - secretRef: name: crowdvision-secrets volumeMounts: - name: application-data mountPath: /app/data resources: requests: cpu: "8" memory: 8Gi nvidia.com/gpu: "1" limits: cpu: "17" memory: 24Gi nvidia.com/gpu: "1" startupProbe: httpGet: path: /api/health/simple port: http timeoutSeconds: 5 failureThreshold: 60 periodSeconds: 10 livenessProbe: httpGet: path: /api/health/simple port: http initialDelaySeconds: 30 periodSeconds: 30 timeoutSeconds: 10 failureThreshold: 5 readinessProbe: httpGet: path: /api/health/simple port: http initialDelaySeconds: 30 periodSeconds: 30 timeoutSeconds: 10 failureThreshold: 5 volumes: - name: application-data persistentVolumeClaim: claimName: crowdvision-data

10.2: Traefik Ingress Manifest (k3s/ingress.yaml)

apiVersion: networking.k8s.io/v1 kind: Ingress metadata: name: crowdvision spec: ingressClassName: traefik rules: - http: paths: - path: /api pathType: Prefix backend: service: name: crowdvision-backend port: name: http - path: /docs pathType: Prefix backend: service: name: crowdvision-backend port: name: http - path: /openapi.json pathType: Prefix backend: service: name: crowdvision-backend port: name: http - path: / pathType: Prefix backend: service: name: crowdvision-frontend port: name: http